# Simultaneous Purification of Genomic DNA and Total RNA from Animal Tissues

# Table of Contents

Important Points Before Starting	2

Required Materials	2

Extraction	5

Total RNA Purification	6

Genomic DNA Purification	7

Troubleshooting	7

# Important Points Before Starting

- Do not allow tissue to thaw during weighing or handling prior to homogenization in Buffer RLT Plus/ß-Mercaptoethanol/Reagent DX mixture. To prevent this, excise and handle tissue in a disposable square weight boat positioned on a dry ice-cooled 3”x6” metal cutting block.

- Ensure usage of a small antistatic polystyrene pour boat and closing of scale walls when weighing tissue to limit error in recording of weight.

- The maximum number of samples to be extracted and purified per person is 6 (i.e., 2 researchers x 6 samples = 12 samples for extraction/purification).

- Homogenized tissue lysates from step 4 can be stored at -70˚C for months.

- Buffer RLT Plus may form a precipitate upon storage. If necessary, redissolve by warming, and, then, place at room temperature.

- Preparation of Buffer RLT Plus/ß-Mercaptoethanol/Reagent DX mixture:

  - 10µL of ß-Mercaptoethanol is added to 1mL of Buffer RLT Plus prior to its use.

  - Reagent DX is added to Buffer RLT Plus at a final concentration of 0.05% (v/v)

    - Reduces homogenization-induced foaming.

- Buffer RPE, Buffer AW1, and Buffer AW2 are, each, supplied as a concentrate. Before using for the first time, add the appropriate volume of 100% ethanol, as indicated on the bottle, to obtain a working solution.

- Buffer RLT Plus, Buffer RW1, and Buffer AW1 contain a guanidine salt and are, therefore, not compatible with disinfecting reagents containing bleach.

- Use 70% ethanol to clean gloves and working area periodically, as RNA purification requires proper aseptic technique for high-quality extracts.

- Place Buffer EB in lab coat pocket prior to beginning procedure to ensure gently warming of solution prior to DNA elution.

- All steps of the procedure, following homogenization, should be performed at room temperature (~25˚C). During so, work quickly.

# Required Materials

- Supplied by Qiagen:

  - AllPrep DNA/RNA Kit (Qiagen, link to product)

  - Reagent DX (Qiagen, link to product)

    - 1.75μL/sample

- Not supplied by Qiagen:

  - Small Antistatic Polystyrene Pour Boats (VWR, SKU#: 76299-240)

  - Disposable Square Weigh Boats (100mL) (VWR, SKU#: 10770-406)

  - Razor Blade (VWR, SKU#: 55411-050)

  - Lysing Matrix D Tubes (VWR, SKU#: IC116913100)

  - Ultrafree® Centrifugal Filter Units (VWR; SKU#: 102970-584)

  - 100% Ethanol (VWR, SKU#: 89125-172)

  - Spray bottle of 70% Ethanol

  - Forceps

  - Punch Biopsy Tool (Fine Science Tools, SKU#: 18035-03)

  - 3”x6” Metal Cutting Block

  - β-Mercaptoethanol (Fisher Scientific, SKU#: ICN19483425)

  - 3x Dry Ice-Filled Containers with Lids

    - 1 for lysing matrix D tubes containing excised tissue

    - 1 for cutting block and disposable square weigh boat

    - 1 for master sample tubes and forceps

  - Pipettes with Corresponding Pipette Tips

  - Microcentrifuge Tube Racks

# Extraction

- Excise 7.0-9.5mg of tissue, from a master sample tube, using forceps and a tissue biopsy tool for sample handling. Denote the exact weight of the excised tissue.

  - NOTE: If necessary, use a 100% ethanol-cleaned razor blade to cut pieces of tissue obtained by the tissue biopsy tool, in a weigh boat on a dry ice-cooled 3”x6” metal cutting block, to reduce sample weight.

  - NOTE: Instruments (e.g., forceps, punch biopsy tool, razor blade) should be cleaned in 100% ethanol prior to initial usage and between excision of different samples.

- Use forceps to submerge the excised tissue in beads within a lysing matrix D tube.

- Add 350µL of Buffer RLT Plus/ß-Mercaptoethanol/Reagent DX mixture to each lysing matrix D tube.

  - NOTE: Ensure Buffer RLT Plus/ß-Mercaptoethanol/Reagent DX mixture does not freeze in dry ice-cooled lysing matrix D tubes prior to homogenization. This is best achieved by indirectly cooling the lysing matrix D tubes in a microcentrifuge tube rack.

- Disrupt and homogenize the lysate at 6m/s for 40 seconds.

  - NOTE: Settings stored in the homogenizer as the Human Kidney protocol.

  - NOTE: If excessive foaming occurs in lysing matrix D tubes, gently spin down for ~10 seconds prior to proceeding to step 5.

- Dump the lysate and beads from the lysing matrix D tubes into centrifugal filter units.

- Centrifuge for 3 minutes at 21,100 x g.

- Carefully transfer supernatant (avoiding pellet) to an AllPrep DNA spin column placed in a 2mL collection tube (supplied).

- Centrifuge for 30 seconds at 10,000 x g.

  - NOTE: Make sure that no liquid remains on the column membrane following centrifugation. If necessary, repeat the centrifugation until all liquid has passed through the membrane.

- Place the AllPrep DNA spin column in a new 2mL collection tube (supplied), and store at room temperature for later DNA purification in steps 18-21. Use the flow-through for RNA purification in steps 10-17.

  - NOTE: Do not store the AllPrep DNA spin column at room temperature for long periods. Do not freeze the column.

# Total RNA Purification

- Add 350µL of 50% ethanol to the flowthrough from step 9, and mix well by pipetting. Do not centrifuge. Proceed immediately to step 11.

  - NOTE: Do not create bubbles/foaming while mixing with the pipette.

- Transfer up to 700µL of the sample, including any precipitate that may have formed, to an RNeasy spin column placed in a 2mL collection tube (supplied), and centrifuge for 15 seconds at 12,000 x g. Discard the flowthrough, and reuse the collection tube in step 12.

- Add 700µL of Buffer RW1 to the RNeasy spin column, and centrifuge for 15 seconds at 12,000 x g. Discard the flowthrough, and reuse the collection tube in step 13.

  - NOTE: After centrifugation, ensure the RNeasy spin column does not contact the flowthrough. Be sure to empty the collection tube completely.

- Add 500µL of Buffer RPE to the RNeasy spin column, and centrifuge for 15 seconds at 8,000 x g. Discard the flowthrough, and reuse the collection tube in step 14.

- Add 500uL of Buffer RPE to the RNeasy spin column, and centrifuge for 2 minutes at 8,000 x g.

- Place the RNeasy spin column in a new 2mL collection tube (supplied). Centrifuge for 1 minute at 21,100 x g.

- Place the RNeasy spin column in a new 1.5mL collection tube (supplied), and add 50µL of RNase-free water directly to the spin column membrane. Allow the RNase-free water to interact with the spin column membrane for 12 minutes, and, then, centrifuge for 1 minute at 8,000 x g.

  - NOTE: Final eluate can be stored at -70˚C for months.

# Genomic DNA Purification

- Add 500µL of Buffer AW1 to the AllPrep DNA spin column from step 9, and centrifuge for 15 seconds at 10,000 x g. Discard the flowthrough, and reuse the spin column in step 18.

- Add 500µL of Buffer AW2 to the AllPrep DNA spin column, and centrifuge for 2 minutes at 21,100 x g.

  - NOTE: After centrifugation, ensure the AllPrep DNA spin column does not contact the flowthrough. If the column contacts the flowthrough, empty the collection tube, and centrifuge the spin column, again, for 1 minute at 21,100 x g.

- Place the AllPrep DNA spin column in a new 1.5mL collection tube (supplied), and add 35µL of Buffer EB directly to the spin column membrane. Allow the Buffer EB to interact with the spin column membrane for 5 minutes, and, then, centrifuge for 1 minute at 8,000 x g.

- Repeat step 19 with 20uL of Buffer EB.

  - NOTE: Final eluate can be stored at -70˚C for months.

# Troubleshooting

- Clogged AllPrep DNA or RNeasy spin column

  - Inefficient disruption and/or homogenization?

    - Increase g-force and/or centrifugation time.

    - Reduce the amount of excised tissue and/or increase homogenization time.

  - Too much starting material?

    - Reduce the amount of excised tissue.

- Low nucleic acid yield

  - Insufficient disruption and/or homogenization?

    - Increase g-force and/or centrifugation time.

    - Reduce the amount of excised tissue and/or increase homogenization time.

    - Increase the amount of lysis buffer (Buffer RLT Plus/ß-Mercaptoethanol/Reagent DX mixture).

  - Too much starting material?

    - Reduce the amount of excised tissue.

  - RNA still bound to RNeasy spin column membrane?

    - Repeat RNA elution (step 16).

  - DNA still bound to AllPrep DNA spin column membrane?

    - Repeat DNA elution, but incubate the AllPrep DNA spin column on the benchtop for 10 minutes with Buffer EB before centrifugation.

    - Heat Buffer EB to 70˚C prior to DNA elution.

  - Ethanol carryover?

    - During the second wash with Buffer RPE (step 14), increase centrifugation time to dry the RNeasy spin column membrane.

- DNA contaminated with RNA

  - Lysate applied to the AllPrep DNA spin column contains ethanol?

    - Add ethanol to the lysate after passing it through the AllPrep DNA spin column.

  - Excised tissue is affecting pH of eluate?

    - pH eluate to ensure it is neutral (~7).

- RNA contaminated with DNA

  - Excised tissue has high DNA content?

    - Reduce the amount of excised tissue.

- Low A260/A280 value in RNA eluate

  - RNase-free water used to dilute RNA for A260/A280 measurement?

    - Use 10mM Tris-Cl, pH 7.5, instead of RNase-free water to dilute the sample prior to examining purity.

- RNA degraded

  - Inappropriate handling of tissue?

    - Ensure that frozen tissue was flash-frozen immediately and stored in liquid nitrogen.

    - Perform all steps prior to homogenization quickly (steps 1-3) to avoid thawing of excised tissue.

  - RNase contamination?

    - Although the AllPrep buffers have been tested and are guaranteed RNase-free, RNases can be introduced during use. Be certain not to introduce RNases during the protocol and/or external handling.

  - Homogenization too vigorous?

    - Reduce homogenization time and/or use a gentler homogenization method.

- Low nucleic acid concentration

  - Elution volume too high?

    - Elute nucleic acids in a smaller volume. Do not use less than 50µL of Buffer EB for the AllPrep DNA spin column, or less than 1x 30µL of RNase-free water for the RNease spin column. Although eluting in a reduced volume will increase nucleic acid concentration, yield may be decreased.

- Poor performance of nucleic acids in downstream experiments

  - Salt carryover during elution?

    - Ensure buffers are at room temperature (~25˚C).

    - Ensure that the correct buffer is used for each step of the protocol.

    - When reusing collection tubes between washing steps, remove residual flow-through from the rim by blotting on clean paper towels.

  - Ethanol carryover?

    - During the second wash with Buffer RPE (step 14), increase centrifugation time to dry the RNeasy spin column membrane.