Tissue preparation: Embedded in optimal cutting temperature compound and cryosectioned (10-μm thickness)

Fixative: 4% (w/v) paraformaldehyde (PFA) in 1× phosphate-buffered saline (PBS)

Temperature: Room temperature unless otherwise specified

Pre-Staining Notes

- Use a syringe filter when applying the SBB solution to prevent precipitate artifacts.

- Use fresh ethanol and xylenes for dehydration and clearing.

- Allow sections to equilibrate to room temperature prior to fixation.

Staining Workflow

| Reagent | Duration | Additional Notes |
| --- | --- | --- |
| 4% (w/v) PFA in 1× PBS | 10 minutes |  |
| 1× PBS | 5 minutes × 3 | On rocker |
| Apply hydrophobic barrier around each section. |  |  |
| 70% Ethanol | 30 seconds |  |
| 0.7% (w/v) SBB in 70% Ethanol | 30 minutes | In dark, humid chamber; apply directly onto sections using syringe filter |
| 70% Ethanol | 3 seconds |  |
| DI Water | 30 seconds × 2 | Gentle agitation in slide holder |
| Nuclear Fast Red | 1 minute |  |
| DI Water | 30 seconds × 2 | Gentle agitation in slide holder |
| 70% Ethanol | 1 minute |  |
| 95% Ethanol | 1 minute |  |
| 100% Ethanol | 1 minute × 2 |  |
| Xylenes | 2 minutes × 2 |  |
| Coverslip with DPX mounting agent and allow to dry overnight. |  |  |

- Place slides in a Coplin jar with 4% (w/v) PFA in 1× PBS. Allow to incubate for 10 minutes.

- Place slides in a Coplin jar with 1× PBS. Allow to incubate on rocker for 5 minutes.

- Repeat Step 2 2× with fresh 1× PBS.

- Apply hydrophobic barrier around each section.

- Place slides in a staining basin with 70% ethanol. Allow to incubate for 30 seconds.

- Place slides in a dark, humid chamber. Apply ~5−10 drops of 0.7% (w/v) SBB in 70% ethanol directly onto sections using a syringe filter.

  - The syringe filter pore size needs to be ≤0.22μm to adequately filter the SBB solution.

- Place slides in a staining basin with 70% ethanol. Allow to incubate for 3 seconds.

- Place slides in a staining basin with DI water. Wash for 30 seconds.

  - Wash the slides by applying gentle agitation to the slide holder.

- Repeat Step 8 1× with fresh DI water.

- Place slides in a staining basin with Nuclear Fast Red. Allow slides to incubate for 1 minute.

- Place slides in a staining basin with DI water. Wash for 30 seconds.

- Repeat Step 11 1× with fresh DI water.

- Place slides in a staining basin with 70% ethanol. Allow to incubate for 1 minute.

- Place slides in a staining basin with 95% ethanol. Allow to incubate for 1 minute.

- Places slides in a staining basin with 100% ethanol. Allow to incubate for 1 minute.

- Repeat Step 15 1× with fresh 100% ethanol.

- Places slides in a staining basin with Xylenes. Allow to incubate for 2 minutes.

- Repeat Step 17 1× with fresh Xylenes.

- Coverslip with DPX mounting agent. Allow to dry overnight.

  - For best results, perform Step 19 while a slide is “shiny” due to Xylenes on its surface.